ForeWatt Technical Report

Electricity Market Forecasting Platform for the Turkish Market

ForeWatt Development Team

January 2026

Abstract

This report presents ForeWatt, an end-to-end electricity market forecasting platform designed for the Turkish electricity market operated by EPIAS. The system provides 24-hour ahead forecasts for both energy consumption and day-ahead market prices using gradient boosting ensemble methods. The consumption model, based on CatBoost with 23 engineered features, achieves a symmetric mean absolute percentage error of 1.95 percent on held-out test data. The price forecasting model employs a hybrid architecture combining CatBoost and LightGBM with adaptive error correction, achieving 11.71 percent sMAPE while operating under a 2-hour data publication delay constraint. The platform is deployed on Google Cloud Run with hourly automated forecast generation, Firestore real-time storage, and a React-based dashboard for visualization. This work demonstrates that carefully engineered gradient boosting models can achieve competitive performance against more complex deep learning approaches while maintaining interpretability and production reliability.

# 1. Introduction

## 1.1 Problem Definition and Motivation

The Turkish electricity market has undergone significant transformation since its liberalization in 2001. The Energy Exchange Istanbul, known as EPIAS, now operates as the central platform for electricity trading in Turkey, handling day-ahead market transactions, intraday trading, and balancing power markets. With an installed capacity exceeding 100 gigawatts and annual electricity consumption surpassing 300 terawatt-hours, the Turkish market represents one of the largest electricity markets in the EMEA region.

Electricity price and consumption forecasting in this market presents unique challenges. The high penetration of renewable energy sources, particularly wind and solar, introduces substantial volatility into both generation patterns and market prices. Turkey's diverse climate zones, spanning Mediterranean, continental, and transitional regions, create complex demand patterns driven by heating and cooling requirements. Furthermore, the market's sensitivity to natural gas prices, hydroelectric reservoir levels, and cross-border electricity flows adds layers of complexity that purely autoregressive models struggle to capture.

A critical operational constraint in the Turkish market is the 2-hour delay in official data publication by EPIAS. Real-time consumption and price data become available only after this lag period, meaning that any forecasting system must produce predictions without access to the most recent two hours of actual observations. This constraint effectively extends the minimum forecast horizon and requires careful feature engineering to bridge the information gap.

The motivation for accurate electricity forecasting extends across multiple stakeholder groups. Generation companies require consumption forecasts to optimize unit commitment and dispatch decisions. Retailers and large consumers need price forecasts to manage procurement strategies and hedge against price volatility. Grid operators rely on demand forecasts for real-time balancing and reserve procurement. The financial implications of forecast errors are substantial, as imbalance penalties in the Turkish market can significantly impact profitability. These considerations motivate the development of ForeWatt as a comprehensive forecasting platform addressing both consumption and price prediction with production-grade reliability.

## 1.2 Project Goals and Objectives

The primary objective of the ForeWatt project is to develop an operational forecasting system capable of generating accurate 24-hour ahead predictions for electricity consumption and day-ahead market prices in the Turkish market. This objective encompasses several specific technical goals that guide the system design and implementation.

The first goal is to achieve forecast accuracy that substantially outperforms naive baseline methods. For consumption forecasting, the target is to surpass the performance of persistence models that simply use the same-hour value from the previous day or week. For price forecasting, the objective is to exceed the accuracy of both persistence baselines and simple autoregressive models. The performance metric selected for evaluation is the symmetric mean absolute percentage error, which provides scale-independent assessment suitable for comparing models across different time periods and market conditions.

The second goal is to design and implement a production-ready system architecture capable of continuous operation with minimal manual intervention. This requires automated data pipelines that handle the complexities of multiple data sources, robust error handling for API failures and data quality issues, and a deployment infrastructure that scales automatically with demand. The selected platform for deployment is Google Cloud Run, which provides serverless container hosting with automatic scaling and integrated scheduling capabilities.

The third goal is to provide accessible forecast visualization through a web-based dashboard. The dashboard should display current forecasts with uncertainty quantification, historical performance tracking, and anomaly detection capabilities. This interface enables stakeholders to consume forecast outputs without requiring technical expertise in data science or programming.

## 1.3 Literature Review and Related Work

Electricity load and price forecasting have been active research areas for several decades, with methodological approaches evolving from statistical time series methods to machine learning and deep learning techniques. This review surveys the relevant literature to contextualize the technical choices made in the ForeWatt system.

Traditional statistical methods for electricity forecasting have centered on autoregressive integrated moving average models and their extensions. ARIMA models capture linear dependencies in time series data through a combination of autoregressive terms, differencing for stationarity, and moving average components. Seasonal ARIMA variants extend this framework to handle the strong daily, weekly, and annual periodicities characteristic of electricity demand. Exponential smoothing methods, including Holt-Winters approaches, provide alternatives that weight recent observations more heavily while explicitly modeling trend and seasonal components. While these methods remain useful as baselines and for interpretability, their linear assumptions limit their ability to capture the complex nonlinear relationships present in electricity markets.

Machine learning methods have demonstrated substantial improvements over traditional statistical approaches for electricity forecasting. Gradient boosting decision tree ensembles, including XGBoost, LightGBM, and CatBoost, have emerged as particularly effective for tabular regression problems with complex feature interactions. These methods construct ensembles of decision trees sequentially, with each tree trained to correct the residual errors of the previous ensemble. The ability to handle heterogeneous features, capture nonlinear relationships, and provide feature importance rankings makes gradient boosting methods well-suited for electricity forecasting applications where domain knowledge can inform feature engineering.

Deep learning approaches have also been extensively explored for time series forecasting. Long short-term memory networks address the vanishing gradient problem in recurrent neural networks, enabling the learning of long-range temporal dependencies. The Transformer architecture, originally developed for natural language processing, has been adapted for time series through models such as the Temporal Fusion Transformer which combines attention mechanisms with interpretable variable selection. More recent architectures including N-BEATS, N-HiTS, and PatchTST have achieved strong performance on benchmark forecasting tasks through various innovations in how temporal patterns are represented and processed.

Despite the architectural sophistication of deep learning methods, empirical studies have repeatedly shown that well-tuned gradient boosting models often match or exceed their performance on electricity forecasting tasks. A comprehensive study by Makridakis and colleagues in the M5 competition demonstrated that gradient boosting methods achieved top performance across diverse forecasting scenarios. Similar findings have been reported specifically for electricity markets, where the structured nature of the problem and the availability of meaningful engineered features favor tree-based methods. The lower computational requirements and greater interpretability of gradient boosting methods provide additional practical advantages for production deployment.

For the specific context of the Turkish electricity market, prior research has examined various forecasting approaches with mixed results. Studies have applied neural networks, support vector machines, and ensemble methods to EPIAS data with varying degrees of success. However, few works have addressed the complete production pipeline from data acquisition through deployment, and most academic studies evaluate performance on historical data without addressing the operational constraints of real-time forecasting with data publication delays.

## 1.4 Approach and Design Philosophy

The design of ForeWatt is guided by several core principles that prioritize production reliability and maintainability alongside forecast accuracy. These principles reflect lessons learned from operational machine learning systems and the specific requirements of electricity market forecasting.

The first principle is the adoption of a medallion data architecture for organizing the data pipeline. Raw data from external sources including EPIAS, weather APIs, and macroeconomic databases is stored in the bronze layer with minimal transformation. The silver layer contains cleaned and validated data with standardized formats and handling of missing values. The gold layer provides feature-engineered datasets ready for model training and inference. This layered architecture enables independent evolution of data sources and models, simplifies debugging of data quality issues, and provides clear interfaces between pipeline stages.

The second principle is the use of ensemble methods with hybrid error correction. Rather than relying on a single model, the price forecasting system combines predictions from multiple gradient boosting algorithms with adaptive error correction mechanisms. This approach improves robustness against model-specific failure modes and enables the system to adapt to changing market conditions without complete retraining. The consumption model uses a single CatBoost model but incorporates uncertainty quantification through conformal prediction to provide reliable prediction intervals.

The third principle is extensive feature engineering leveraging domain knowledge of electricity markets. Features are constructed to capture known drivers of consumption and prices, including temperature-based heating and cooling degree days, calendar effects such as holidays and weekends, lagged values at intervals corresponding to daily and weekly seasonality, and market-derived signals such as generation mix and capacity margins. This domain-informed approach to feature engineering enables simpler models to achieve strong performance by presenting relevant information in forms that tree-based learners can effectively exploit.

The fourth principle is prioritizing production considerations throughout the design process. All components are designed for automated operation with comprehensive logging and monitoring. The system handles missing data gracefully through forward-filling and fallback strategies rather than failing on incomplete inputs. The deployment architecture on Google Cloud Run provides automatic scaling, built-in load balancing, and integration with Cloud Scheduler for hourly forecast generation. This production focus ensures that the system delivers value continuously rather than serving only as a research prototype.

The remainder of this report is organized as follows. Section 2 describes the data sources and preprocessing pipeline in detail. Section 3 presents the feature engineering methodology and the complete feature sets used by each model. Section 4 covers the model architectures, training procedures, and hyperparameter optimization. Section 5 reports experimental results including accuracy metrics and comparison with baselines. Section 6 describes the system architecture and deployment infrastructure. Section 7 concludes with a discussion of limitations and directions for future work.

## 2.1 Data Collection and Data System

The ForeWatt forecasting platform was built upon a comprehensive data collection infrastructure designed to capture the multifaceted factors influencing electricity consumption and prices in the Turkish market. The data system integrates information from three primary sources: the Turkish Electricity Market Transparency Platform (EPIAS), the Open-Meteo Weather API, and the Electronic Data Delivery System (EVDS) of the Central Bank of the Republic of Turkey. This section describes the data sources, collection methodologies, and the medallion architecture employed for data processing and storage.

The primary data source for electricity market variables was the EPIAS Transparency Platform, accessed through the eptr2 Python library with authenticated API credentials. The EPIAS data pipeline was designed to fetch twelve distinct datasets covering the full spectrum of market operations. Real-time consumption data, representing the actual hourly electricity load measured by TEIAS (Turkish Electricity Transmission Corporation), served as the target variable for consumption forecasting with values typically ranging between 20,000 and 50,000 MW. Day-ahead consumption forecasts published by TEIAS provided baseline predictions against which model improvements could be measured. Market price data included the Day-Ahead Market Clearing Price (PTF), the System Marginal Price (SMF) from the balancing power market, the Intraday Market weighted average price, and the overall Weighted Average Price across all market segments. Generation data encompassed real-time generation by source and plant, available capacity declarations (EAK), day-ahead generation plans (KGUP), and bilateral contract plans (KUDUP). Wind generation forecasts from the RITM system and hydroelectric reservoir status from DSI (State Hydraulic Works) completed the EPIAS data collection, providing supply-side fundamentals critical for price forecasting.

Meteorological data was sourced from the Open-Meteo Weather API, which provides free access to historical and forecast weather information with hourly resolution. A demand-side weather pipeline was implemented to fetch data for the ten largest Turkish cities by population, collectively representing approximately 49 percent of the national population. The selected cities included Istanbul with a population weight of 18.3 percent, Ankara at 6.9 percent, Izmir at 5.2 percent, Bursa at 3.8 percent, Antalya at 3.2 percent, Konya at 2.7 percent, Adana at 2.7 percent, Sanliurfa at 2.6 percent, Gaziantep at 2.6 percent, and Kocaeli at 2.5 percent. Eight weather variables were collected for each city: temperature at 2 meters height, relative humidity, precipitation amount, rainfall specifically, cloud cover percentage, wind speed at 10 meters, surface atmospheric pressure, and apparent temperature representing the perceived temperature accounting for humidity and wind effects. Population-weighted national aggregates were computed from city-level data to create representative weather features for the entire country. The API implementation incorporated automatic request caching and exponential backoff retry logic to handle rate limiting, with a 7-second delay between city requests to respect API usage policies.

Macroeconomic indicators were obtained from the EVDS system operated by the Central Bank of the Republic of Turkey. Given that Turkey imports approximately 70 percent of its energy requirements, primarily natural gas and oil, exchange rate fluctuations have a substantial impact on electricity generation costs and market prices. The fetched series included the Consumer Price Index (TUFE) and Producer Price Index (UFE) for inflation monitoring, the M2 money supply as a monetary policy indicator, and Turkish Lira deposit interest rates. Additionally, daily foreign exchange rates were collected including USD/TRY, EUR/TRY, and gold prices in Turkish Lira (XAU/TRY). Derived features were computed from the raw exchange rate data, including a weighted FX basket calculated as the simple average of USD and EUR rates, 7-day and 30-day momentum indicators representing the rate of change, and a 30-day rolling standard deviation capturing exchange rate volatility. Daily FX data was converted to hourly frequency through forward-fill replication to enable alignment with the hourly electricity market data.

Data management followed the medallion architecture pattern, organizing data into three progressively refined layers: Bronze, Silver, and Gold. The Bronze layer stored raw data exactly as received from source APIs, preserving the original format and enabling reproducibility. Separate subdirectories maintained data from each source including EPIAS market data, demand-side weather observations, macroeconomic indicators, and calendar information. The Silver layer contained cleaned and validated data after applying quality assurance transformations. Validation procedures included duplicate record removal, timestamp chronological ordering verification, range validation to identify physically implausible values such as consumption below 10,000 MW or above 60,000 MW, and flagging of columns with greater than 10 percent missing values. All datetime columns were standardized to the Europe/Istanbul timezone to ensure temporal consistency across datasets. The Gold layer housed feature-engineered datasets ready for model training, including lag features at 24-hour, 48-hour, and 168-hour intervals, rolling statistics such as means and standard deviations, and the master dataset that merged all data sources into a unified feature matrix.

Figure 1. Medallion data architecture showing Bronze, Silver, and Gold layers with data flow from external sources through cleaning and feature engineering stages.

All data was persisted in Apache Parquet format with Snappy compression, selected for its efficient columnar storage, fast read performance, and native support for complex data types including timezone-aware timestamps. Files were organized with monthly partitioning for forecast archives and date-range naming conventions for training datasets. Dual-format export to CSV was maintained as a secondary format for human readability and compatibility with spreadsheet applications. The complete master dataset spanning from January 2020 to October 2025 contained approximately 51,000 hourly records with 80 features, stored in a 52.5 megabyte Parquet file. The data pipeline supported incremental updates through skip-existing logic that detected previously downloaded data and avoided redundant API calls, significantly reducing fetch times for routine updates while ensuring data completeness for model retraining.

## 2.2 Feature Engineering and Master Data

The transformation of raw data into predictive features represents a critical stage in the ForeWatt pipeline. A comprehensive feature engineering framework was developed to extract meaningful signals from heterogeneous data sources, including historical consumption patterns, electricity market dynamics, meteorological observations, and calendar information. The feature engineering pipeline is implemented through a modular architecture consisting of specialized generators for lag features, rolling window statistics, calendar encodings, and weather-derived indicators. These components operate on data organized according to a medallion architecture, processing information from the Silver layer (cleaned and normalized) to produce Gold layer assets (machine learning-ready features).

Lag features constitute the foundation of the autoregressive modeling approach employed in ForeWatt. For the consumption forecasting model, lags are computed at multiple temporal horizons to capture patterns operating at different time scales. Short-term lags at 1, 2, 3, and 6 hours capture immediate consumption inertia and autoregressive dependencies that reflect the physical constraints of electricity demand. The 12-hour and 24-hour lags capture diurnal patterns and day-over-day changes, while the 48-hour and 168-hour (weekly) lags encode longer-term seasonal cycles. Temperature lag features are similarly constructed at 1, 2, 3, 24, and 168 hour intervals to account for the delayed response of electricity demand to weather conditions. Price lag features at 24-hour and 168-hour horizons provide economic context by reflecting historical market conditions.

Rolling window statistics extend the feature set by computing aggregate measures over sliding windows of 24 hours and 168 hours. For each window size, the mean, standard deviation, minimum, and maximum values are calculated for consumption, temperature, and price variables. These statistics capture both the central tendency and variability of the underlying processes. Derived features are computed from these rolling statistics, including the consumption range (maximum minus minimum) as a measure of daily volatility, the coefficient of variation (standard deviation divided by mean) as a normalized variability indicator, and the diurnal temperature range to characterize weather stability. The rolling standard deviation of price over 24 hours serves as a proxy for market volatility, which is particularly important for price prediction during periods of supply uncertainty.

Calendar features encode temporal structure and institutional patterns that influence electricity consumption and prices. Basic temporal features include day of week, day of month, month, and week of year. Binary indicators distinguish weekends from weekdays, reflecting the systematic reduction in commercial and industrial demand during non-working days. Holiday features are constructed from a comprehensive calendar of Turkish official holidays, religious observances, and half-day holidays. The holiday encoding includes both day-level and hour-level flags, with special handling for half-day holidays where morning hours may exhibit normal demand patterns. Cyclical encodings using sine and cosine transformations are applied to day of week and month features to preserve the periodic nature of these variables and avoid artificial discontinuities at period boundaries.

Weather features are derived from temperature, humidity, and wind speed observations collected from twelve major Turkish cities. A population-weighted national temperature index is computed to represent aggregate weather conditions across the country. Heating Degree Days (HDD) and Cooling Degree Days (CDD) are calculated using standard formulas with base temperatures calibrated for Turkish climate conditions. The heat index combines temperature and humidity to characterize perceived temperature during summer months when air conditioning loads are significant. Binary indicators flag extreme temperature conditions (hot and cold thresholds) that trigger non-linear demand responses. These weather features capture the thermosensitive component of electricity demand, which can account for substantial variation during extreme weather events.

Figure 2. Machine learning model architecture showing feature flow and model ensemble

The price forecasting model employs a distinct feature set designed to capture market dynamics and supply-demand fundamentals. Market signal features include thermal gap, which measures the difference between thermal generation capacity and actual thermal output, indicating the availability of dispatchable generation. Renewable saturation captures the proportion of demand met by variable renewable sources, which influences price volatility through merit order effects. The spark spread proxy approximates the profitability of gas-fired generation using historical price relationships. System short signals indicate periods when scheduled generation may be insufficient to meet forecasted demand, typically associated with price spikes. Load factor and reserve margin ratio characterize system utilization and capacity adequacy, respectively.

Profile evolution features capture the time-varying nature of hourly price patterns. The daily average price and hourly ratio relative to the daily average encode the typical shape of the daily price curve. Profile features computed over 14-day and 28-day windows capture the recent evolution of these patterns, while profile momentum measures the rate of change in the profile shape. Solar-specific features account for the growing penetration of photovoltaic generation in Turkey, including solar ratio, solar profiles, and price-solar interaction terms. These features enable the model to adapt to structural changes in the price formation process as the generation mix evolves.

Table 1. Consumption Model Features (23 total)

| Feature Name | Description |
| --- | --- |
| consumption_lag_24h | Consumption value from 24 hours prior |
| consumption_lag_48h | Consumption value from 48 hours prior |
| consumption_lag_168h | Consumption value from one week prior |
| consumption_rolling_mean_24h | 24-hour rolling mean of consumption |
| consumption_rolling_std_24h | 24-hour rolling standard deviation |
| temp_national | Population-weighted national temperature |
| humidity_national | Population-weighted national humidity |
| HDD | Heating Degree Days |
| CDD | Cooling Degree Days |
| heat_index | Combined temperature-humidity index |
| is_hot | Binary indicator for hot conditions |
| is_cold | Binary indicator for cold conditions |
| temp_lag_24h | Temperature from 24 hours prior |
| hour_sin, hour_cos | Cyclical encoding of hour of day |
| dow_sin, dow_cos | Cyclical encoding of day of week |
| month_sin, month_cos | Cyclical encoding of month |
| is_weekend | Binary weekend indicator |
| is_holiday_day | Binary day-level holiday indicator |
| is_holiday_hour | Binary hour-level holiday indicator |
| price_ptf_lag_24h | Day-ahead price from 24 hours prior |

Table 2. Price Model Features (33 total)

| Feature Name | Description |
| --- | --- |
| hour_sin, hour_cos | Cyclical encoding of hour of day |
| dow_sin, dow_cos | Cyclical encoding of day of week |
| is_weekend | Binary weekend indicator |
| price_ptf_rolling_std_24h | 24-hour rolling price volatility |
| price_ptf_rolling_mean_24h | 24-hour rolling price average |
| price_ptf_rolling_min_24h | 24-hour rolling price minimum |
| price_ptf_rolling_max_24h | 24-hour rolling price maximum |
| price_ptf_lag_24h | Day-ahead price from 24 hours prior |
| price_ptf_lag_168h | Day-ahead price from one week prior |
| thermal_gap | Thermal capacity minus thermal generation |
| thermal_gap_lag_24h | Thermal gap from 24 hours prior |
| renewable_saturation | Renewable generation as fraction of demand |
| spark_spread_proxy_lag_24h | Historical spark spread proxy |
| system_short_signal | Binary indicator for system shortage |
| load_factor | Actual demand divided by peak capacity |
| consumption_forecast | Forecasted consumption for target hour |
| reserve_margin_ratio | Available reserve as fraction of demand |
| price_volatility_lag24h | Lagged price volatility measure |
| realtime_premium_lag24h | Lagged real-time price premium |
| hour | Hour of day (integer) |
| daily_avg_price | Daily average price level |
| hourly_ratio | Current hour price to daily average ratio |
| profile_14d, profile_28d | Hourly price profiles over rolling windows |
| profile_momentum | Rate of change in price profile |
| daily_avg_momentum | Rate of change in daily average price |
| solar_ratio | Solar generation as fraction of capacity |
| solar_profile_14d, solar_profile_28d | Solar generation profiles |
| solar_momentum | Rate of change in solar profile |
| price_solar_interaction | Interaction between price and solar features |

The master dataset is produced by merging all feature components through a systematic process implemented in the feature merger module. Features from different sources are aligned on timestamp and joined using inner merges to ensure temporal consistency. The resulting master dataset is stored in the Gold layer at data/gold/master/ in Apache Parquet format for efficient storage and retrieval. The current production version (v2) contains approximately 52.5 megabytes of data spanning from January 2020 to October 2025, comprising over 50,000 hourly observations with all engineered features. A versioning scheme based on creation date and feature hash ensures reproducibility and enables tracking of dataset evolution across model iterations. Accompanying metadata files in JSON format document the feature list, data date range, missing value statistics, and creation timestamp for each version of the master dataset.

## 2.3 Model Selection

A comprehensive evaluation of deep learning architectures was conducted to assess their suitability for electricity market forecasting in the Turkish context. A total of 90 experiments were performed across three state-of-the-art transformer-based models: Temporal Fusion Transformer (TFT), Neural Hierarchical Interpolation for Time Series (N-HITS), and PatchTST. These architectures were selected due to their documented success in various time series forecasting benchmarks and their ability to capture complex temporal dependencies. The experimental framework encompassed both price and consumption forecasting tasks, with hyperparameter optimization performed through systematic grid search across learning rates, hidden dimensions, attention heads, and dropout rates. The complete experimental suite required 1.67 hours of computation time on GPU-accelerated infrastructure.

The experimental results revealed significant performance variations across model architectures and forecasting targets. For electricity price forecasting, the N-HITS architecture achieved the best performance among deep learning models with a symmetric Mean Absolute Percentage Error (sMAPE) of 16.01% and a Mean Absolute Scaled Error (MASE) of 0.67. The TFT model followed closely with a sMAPE of 16.79%, while PatchTST exhibited the highest error at 17.17%. For consumption forecasting, the ranking differed notably, with TFT achieving the lowest sMAPE of 11.07%, followed by N-HITS at 12.22% and PatchTST at 12.40%. These results are summarized in Table 3, which presents the performance metrics for each deep learning architecture across both forecasting tasks.

Table 3. Deep Learning Model Performance Comparison

| Model | Price sMAPE | Consumption sMAPE | Price MASE | Consumption MASE |
| --- | --- | --- | --- | --- |
| N-HITS | 16.01% | 12.22% | 0.67 | 2.59 |
| TFT | 16.79% | 11.07% | 0.69 | 2.33 |
| PatchTST | 17.17% | 12.40% | 0.72 | 2.63 |

A critical examination of the MASE values revealed substantial evidence of overfitting in the consumption forecasting models. The MASE metric, which compares model performance against a naive seasonal baseline, indicated that all deep learning models performed significantly worse than this simple benchmark for consumption prediction. Specifically, MASE values exceeding 1.0 signify that the model fails to outperform a naive forecast that simply repeats the value from the previous seasonal period. With consumption MASE values ranging from 2.33 to 2.63 across all architectures, the deep learning models demonstrated performance degradation of 133% to 163% compared to the naive baseline. This pattern strongly suggests that the models memorized training data patterns rather than learning generalizable temporal relationships.

The observed overfitting behavior was attributed primarily to the limited volume of training data available for the Turkish electricity market. The dataset encompassed approximately four to five years of hourly observations, which, while substantial for traditional machine learning approaches, proved insufficient for the high-capacity deep learning architectures evaluated. Transformer-based models typically require extensive training corpora to effectively learn attention patterns and temporal dependencies without resorting to memorization. The significant gap between training and test performance metrics, particularly for consumption forecasting, confirmed that the models were unable to generalize beyond the training distribution. Furthermore, the relatively stable consumption patterns in the training period did not adequately prepare the models for the variability encountered in the test set.

Based on the comprehensive analysis of deep learning performance limitations, gradient boosting methods were selected as the primary modeling approach for the ForeWatt platform. The CatBoost algorithm was adopted for consumption forecasting, achieving a sMAPE of 1.95% on the test set, which represents a 5.7-fold improvement over the best deep learning result of 11.07% obtained by TFT. This dramatic performance enhancement demonstrated the superior ability of gradient boosting to extract predictive signals from limited training data while maintaining robust generalization to unseen periods. The ensemble nature of gradient boosting, which combines numerous weak learners through sequential optimization, provided an effective regularization mechanism that prevented the overfitting observed in deep learning approaches.

For price forecasting, a hybrid ensemble combining CatBoost and LightGBM was implemented, achieving a sMAPE of 11.71% compared to the N-HITS result of 16.01%, representing a 27% relative improvement. The hybrid architecture leveraged the complementary strengths of both gradient boosting frameworks, with CatBoost providing robust handling of categorical features and LightGBM contributing efficient leaf-wise tree growth. Beyond raw predictive accuracy, the gradient boosting approach offered several practical advantages critical for production deployment. Training time was reduced from hours to minutes, enabling rapid model iteration and retraining as new data became available. Inference latency decreased substantially, supporting the real-time forecasting requirements of the hourly prediction pipeline.

An additional consideration in the model selection decision was the interpretability of gradient boosting methods compared to deep learning architectures. The feature importance scores generated by CatBoost and LightGBM provided transparent insights into the factors driving price and consumption predictions, facilitating model validation by domain experts and enabling targeted feature engineering improvements. This interpretability was deemed essential for building trust with stakeholders in the Turkish electricity market and for diagnosing prediction errors in production. The combination of superior predictive performance, computational efficiency, and model interpretability established gradient boosting as the optimal approach for the ForeWatt forecasting platform, with deep learning architectures reserved for potential future investigation as training data volumes increase.

## 2.4 Current Price and Consumption Models' Experiments and Results

This section presents the comprehensive experimental evaluation conducted to develop and optimize the ForeWatt forecasting models. A systematic approach was employed to investigate the impact of training data volume, feature engineering decisions, model architecture choices, and hyperparameter configurations on prediction accuracy. The experiments were conducted using a rigorous train-validation-test split methodology, with the test period spanning from June 2024 to October 2025, comprising 12,429 hours of unseen data. The symmetric Mean Absolute Percentage Error (sMAPE) was selected as the primary evaluation metric due to its bounded nature and stability when dealing with values approaching zero, which is particularly relevant for electricity price forecasting where near-zero prices occasionally occur.

### 2.4.1 Consumption Model Experiments

The first experimental dimension investigated was the impact of training data volume on consumption prediction accuracy. Four configurations were evaluated, ranging from one year to four years of historical data. The results demonstrated a clear pattern of diminishing returns as additional training data was incorporated. Training with one year of data (2023) yielded a sMAPE of 2.27%, which improved to 2.16% with two years of data. The inclusion of a third year further reduced the error to 2.08%, while extending to four years of training data produced only a marginal improvement to 2.07%. This analysis revealed that consumption patterns in the Turkish electricity market are relatively stable over time, with three to four years of historical data providing sufficient coverage of seasonal variations without introducing excessive noise from outdated patterns.

Figure 3. Impact of training data size on consumption model performance.

A feature ablation study was conducted to understand the contribution of each feature group to the overall prediction accuracy. The consumption model utilizes 23 features organized into three primary groups: lag-based features capturing historical consumption patterns, weather-related features encoding meteorological conditions, and calendar features representing temporal patterns. When evaluated in isolation, lag features alone achieved a sMAPE of 3.03%, demonstrating their fundamental importance in capturing the autoregressive nature of electricity consumption. Weather features alone proved insufficient, yielding a sMAPE of 12.95%, while calendar features alone achieved 10.61%. The most significant finding was that combining lag and calendar features produced the best performance at 1.96% sMAPE, outperforming the complete feature set (2.07%). This counterintuitive result suggests that weather information may already be implicitly encoded within the historical consumption patterns, and its explicit inclusion introduces additional noise that marginally degrades prediction accuracy.

Figure 4. Feature importance ranking for the consumption forecasting model.

Hyperparameter optimization was performed on the CatBoost model to identify the optimal configuration. The baseline model employed a learning rate of 0.03, tree depth of 5, and 1,000 iterations. Various configurations were systematically evaluated, including adjustments to tree depth (3, 5, and 8), learning rate (0.01, 0.03, and 0.10), number of iterations (1,000 and 2,000), and regularization parameters. The experiments revealed that a higher learning rate of 0.10 achieved the best performance, reducing sMAPE from 2.07% to 1.95%. Deeper trees (depth 8) provided slight improvements but at the cost of increased computational complexity, while shallower trees (depth 3) exhibited underfitting with a sMAPE of 2.33%. The final optimized consumption model achieved a sMAPE of 1.95%, corresponding to a Mean Absolute Error (MAE) of 808.5 MWh and an R-squared value of 0.969, indicating that the model explains 96.9% of the variance in actual consumption values.

Figure 5. Consumption model predictions versus actual values over the test period.

### 2.4.2 Price Model Experiments

The price model exhibited different characteristics regarding training data requirements compared to the consumption model. Unlike consumption patterns, which demonstrated diminishing returns after three years, price prediction continued to benefit from additional historical data across all tested configurations. Training with one year of data produced a sMAPE of 12.34%, which unexpectedly increased slightly to 12.43% with two years, suggesting that the two-year period may have captured an anomalous market regime. However, extending to three years reduced the error to 11.82%, and the full four-year dataset achieved the best performance at 11.71% sMAPE. This finding indicates that electricity prices in the Turkish market exhibit longer-term dependencies and regime changes that require extended historical context to model effectively.

A comprehensive evaluation of model architectures was conducted to determine the optimal approach for price forecasting. Three configurations were compared: CatBoost alone, LightGBM alone, and an ensemble combining both algorithms. CatBoost achieved a sMAPE of 11.94%, demonstrating its strength as a single model, while LightGBM produced a sMAPE of 12.46% with significantly faster training times. The ensemble approach, which combines predictions from both models using optimized weights of 61.4% CatBoost and 38.6% LightGBM, achieved the best performance at 11.71% sMAPE. The ensemble's superiority can be attributed to the complementary error patterns of the two algorithms, with each model capturing different aspects of the price dynamics, thereby reducing the overall prediction variance when combined.

Figure 6. Comparison of price model architectures: CatBoost, LightGBM, and Ensemble.

A novel error correction framework was developed and evaluated to address systematic prediction biases in the price model. Four configurations were compared: raw ensemble predictions without correction, Simple Adaptive Error Correction (AEC) that learns systematic biases from validation errors, K-Nearest Neighbors Error Correction (KNN-EC) that leverages similar historical contexts to estimate expected errors, and a hybrid approach combining both methods with equal weighting. The raw ensemble achieved a sMAPE of 12.58%, which was reduced to 11.95% with Simple AEC and 11.94% with KNN-EC. The hybrid approach, combining 50% Simple AEC and 50% KNN-EC corrections, achieved the best performance at 11.71% sMAPE, representing a 6.9% relative improvement over the uncorrected predictions. This hybrid methodology was designated as CHybrid V14 and was selected as the final production model.

Figure 7. CHybrid error correction architecture combining Simple AEC and KNN-EC methods.

Figure 8. Feature importance ranking for the price forecasting model.

Unlike the consumption model where weather features degraded performance, the price model benefited from all 33 features in its feature set. Feature ablation revealed that price lag features alone achieved a sMAPE of 13.51%, while market signal features alone performed better at 12.84%, indicating the high predictive value of features such as thermal gap, renewable saturation, and spark spread proxy. Calendar features contributed a sMAPE of 13.00% when used in isolation. Combining price lags with market signals improved performance to 12.13%, and the complete feature set achieved the best result at 11.71%. These findings highlight the importance of incorporating domain-specific market fundamentals for electricity price forecasting.

Figure 9. Price model predictions versus actual values over the test period.

### 2.4.3 Baseline Comparisons

To establish the value of the machine learning approach, both models were compared against standard baseline methods commonly employed in time series forecasting. For consumption prediction, the naive baseline using the previous day's value at the same hour (Lag-24h) achieved a sMAPE of 5.69%, while the seasonal naive baseline using the previous week's value (Lag-168h) achieved 5.48%. Rolling mean and hourly mean baselines performed significantly worse at 15.22% and 13.37% respectively. The optimized consumption model at 1.95% sMAPE represents a 2.8-fold improvement over the best baseline. For price prediction, the naive baseline achieved 21.96% sMAPE and the seasonal naive achieved 21.60%, while rolling mean and hourly mean baselines yielded 25.38% and 42.16% respectively. The price model at 11.71% sMAPE represents a 1.8-fold improvement over the seasonal naive baseline. These results demonstrate the substantial value of the machine learning approach for both consumption and price forecasting in the Turkish electricity market.

Table 4. Baseline comparison for consumption and price models.

| Method | Consumption sMAPE | Price sMAPE | Improvement Factor |
| --- | --- | --- | --- |
| Naive (Lag-24h) | 5.69% | 21.96% | Baseline |
| Seasonal Naive (Lag-168h) | 5.48% | 21.60% | Baseline |
| Rolling Mean (24h) | 15.22% | 25.38% | - |
| Hourly Mean | 13.37% | 42.16% | - |
| Our ML Model | 1.95% | 11.71% | 2.8x / 1.8x |

### 2.4.4 Final Model Performance

The final optimized models demonstrate excellent performance across all evaluation metrics. The consumption model achieves a test sMAPE of 1.95%, a MAE of 808.5 MWh, and an R-squared of 0.969, indicating highly accurate predictions with 96.9% of variance explained. The error distribution analysis reveals that 50% of predictions have an absolute error of 536.5 MWh or less, and 90% have an error of 1,825.7 MWh or less. The slight negative Mean Bias Error of -341.6 MWh indicates a minor tendency toward under-prediction. The price model achieves a test sMAPE of 11.71%, a MAE of 48.2 TL/MWh, and an R-squared of 0.871. Price prediction is inherently more challenging due to market volatility, but 90% of predictions fall within 113 TL/MWh of the actual price. The hybrid error correction approach successfully reduced systematic biases, achieving a final sMAPE that surpasses the theoretical oracle floor of 11.75%, demonstrating the effectiveness of the developed methodology.

Table 5. Final consumption model performance metrics.

| Metric | Value | Interpretation |
| --- | --- | --- |
| sMAPE | 1.95% | Primary accuracy metric |
| MAE | 808.5 MWh | Average absolute error |
| RMSE | 1,194.8 MWh | Penalizes large errors |
| MASE | 0.76 | Better than naive (< 1.0) |
| R-squared | 0.969 | 96.9% variance explained |
| MBE | -341.6 MWh | Slight under-prediction |

Table 6. Final price model performance metrics.

| Metric | Value | Interpretation |
| --- | --- | --- |
| sMAPE | 11.71% | Primary accuracy metric |
| MAE | 48.2 TL/MWh | Average absolute error |
| RMSE | 69.0 TL/MWh | Penalizes large errors |
| MASE | 0.36 | Better than naive (< 1.0) |
| R-squared | 0.871 | 87.1% variance explained |
| MBE | +2.5 TL/MWh | Slight over-prediction |

Table 7. Best model performance summary.

| Model | Configuration | Test sMAPE | Test MAE | Test R-squared |
| --- | --- | --- | --- | --- |
| Consumption | CatBoost V2, lr=0.1 | 1.95% | 808.5 MWh | 0.969 |
| Price | CHybrid V14, 4yr data | 11.71% | 48.2 TL/MWh | 0.871 |

In summary, the experimental evaluation has yielded production-ready forecasting models for both electricity consumption and price prediction. The consumption model benefits from a streamlined feature set combining lag and calendar features with a higher learning rate, while the price model requires a more comprehensive approach incorporating market fundamentals, ensemble learning, and hybrid error correction. Both models significantly outperform standard baseline methods, demonstrating the value of the developed machine learning pipeline for electricity market forecasting in Turkey. The test period of 17 months (12,429 hours) provides robust evidence of model generalization across varying market conditions, seasonal patterns, and demand profiles.

## 2.5 System Architecture

The ForeWatt platform was designed with a cloud-native architecture to ensure scalability, reliability, and cost-effectiveness for production deployment. The system architecture follows modern microservices principles while maintaining simplicity through a serverless deployment model. The backend API was built using FastAPI, a high-performance Python web framework that leverages asynchronous programming patterns for optimal throughput, and is deployed on Google Cloud Run, which provides automatic scaling, pay-per-use pricing, and seamless integration with other Google Cloud services. The architecture was designed to handle the complete lifecycle of electricity market forecasting, from data ingestion through external APIs to real-time forecast delivery to end users.

Figure 10. System context diagram illustrating the high-level interactions between ForeWatt and external systems including EPIAS, weather APIs, and end users.

Figure 11. Complete system architecture showing data flow from external sources through the ML pipeline to the frontend dashboard.

The FastAPI application serves as the central API gateway, exposing multiple endpoints for forecast generation, data retrieval, and system monitoring. The primary endpoints include POST /forecast for triggering hourly forecast generation, GET /api/realtime/{model} for fetching real-time forecasts from Firestore, GET /history/{model} for retrieving historical data from Parquet archives, and GET /api/anomaly/{model} for accessing anomaly detection results. Additional endpoints such as GET /api/aggregates/day-type/{model} and GET /api/aggregates/hourly/{model} provide pre-computed statistics for efficient dashboard loading, enabling rapid visualization of weekday versus weekend consumption patterns and hourly demand profiles. The API was designed with RESTful principles, utilizing query parameters for date filtering and pagination while returning JSON responses with consistent data structures that facilitate seamless integration with the React frontend.

Figure 12. FastAPI endpoint architecture showing the routing structure and handler organization for forecast and data retrieval operations.

Figure 13. Cloud deployment architecture on Google Cloud Platform showing the interaction between Cloud Run, Cloud Scheduler, Firestore, and Cloud Storage.

To optimize cold start performance on Cloud Run, several strategies were implemented to minimize container initialization latency. The forecast pipeline and Firestore client were designed with lazy loading patterns, where expensive imports and model loading are deferred until the first request requires them. This approach reduces the initial container startup time from approximately 15 seconds to under 3 seconds, significantly improving user experience for the first request after container scaling events. GZIP compression middleware was enabled with a minimum response size of 1000 bytes, achieving 80-90 percent reduction in response payload sizes for large forecast datasets containing multiple days of hourly predictions. Cross-Origin Resource Sharing (CORS) middleware was configured to allow requests from the Firebase-hosted dashboard domain while maintaining security for production deployments by rejecting unauthorized origins.

Figure 14. Backend services dataflow diagram depicting the movement of data between API handlers, storage layers, and external data sources.

The application was containerized using Docker with a Python 3.11-slim base image to minimize the container footprint while providing all necessary dependencies for machine learning inference operations. The Dockerfile was structured to leverage layer caching by copying requirements.txt separately before the application code, ensuring that dependency installation is cached across builds and reducing deployment times during iterative development cycles. System dependencies including build-essential and libgomp1 were installed to support the CatBoost and LightGBM machine learning libraries, which require OpenMP for parallel tree evaluation. The trained model files were bundled directly into the container image to eliminate model loading latency during inference, with the models directory containing the CatBoost and LightGBM model artifacts for both price and consumption forecasting at a combined size of approximately 15 megabytes.

Figure 15. Sequence diagram illustrating the hourly forecast generation flow triggered by Cloud Scheduler.

Figure 16. Detailed dataflow sequence for hourly forecast generation showing data fetching, feature engineering, and prediction steps.

Google Cloud Scheduler was configured to trigger the forecast pipeline every hour by sending a POST request to the /forecast endpoint with appropriate authentication headers. Upon receiving the trigger, the ForecastPipeline class orchestrates the complete forecasting workflow through a series of coordinated steps. The pipeline first loads the latest market data from the master Parquet file stored in Google Cloud Storage, then fetches real-time EPIAS data for the most recent hours to ensure predictions incorporate the latest market conditions. Feature engineering transformations are applied to construct the input vectors required by the trained models, and 24-hour ahead forecasts are generated using both the price and consumption models. The forecast results were stored in multiple locations to optimize for different access patterns. Google Cloud Firestore was used for real-time data access, with forecasts stored under the forecasts/latest/{type}/current document path for immediate retrieval by the dashboard with sub-millisecond read latency.

Figure 17. Sequence diagram for real-time data retrieval showing the interaction between frontend, API, and Firestore.

Figure 18. Multi-layer caching architecture sequence showing cache lookup, population, and invalidation patterns.

The storage architecture followed a medallion pattern with bronze, silver, and gold data layers to ensure data quality and traceability throughout the processing pipeline. The bronze layer contained raw data fetched from EPIAS, Open-Meteo, and EVDS APIs in their original formats with timestamps recording ingestion time. The silver layer stored cleaned and validated data with standardized schemas, including null handling, outlier filtering, and timezone normalization to Europe/Istanbul. The gold layer housed the feature-engineered master dataset used for model training and inference, stored as master_v2_fundamental.parquet with a total size of approximately 52.5 megabytes containing over two years of hourly observations. Historical forecasts were archived in Parquet format organized by month using the pattern data/forecasts/{type}/{YYYY-MM}.parquet for efficient batch processing and backtesting operations. The ForecastStorage class provided abstraction over the storage layer, implementing methods for saving forecasts with automatic monthly partitioning and retrieving historical forecasts with date range filtering. Query result caching with a 30-minute time-to-live was implemented using an in-memory LRU cache to reduce redundant data fetching and improve API response times for frequently accessed endpoints such as the real-time forecast view.

For local development and testing, a Docker Compose configuration was provided that orchestrates multiple services including the scheduler, API, dashboard, MLflow for experiment tracking, and InfluxDB for optional time-series storage with retention policies. The scheduler service was configured to run the forecast pipeline at configurable intervals specified through environment variables, enabling rapid iteration on the forecasting logic without requiring cloud deployment. The MLflow service provided a web interface for tracking model experiments and comparing performance across different model versions, storing artifacts locally or in cloud storage depending on configuration. This development environment enabled rapid iteration on the forecasting models and API endpoints before deployment to the production Cloud Run environment, with environment parity ensuring consistent behavior between local and production systems.

## 2.6 Frontend & Dashboard

The ForeWatt dashboard was developed as a modern single-page application using React 19 with TypeScript for type safety and improved developer experience through compile-time error detection and enhanced IDE support. The build system was configured with Vite, a next-generation frontend tooling framework that provides fast hot module replacement during development with sub-second update times and optimized production builds with tree-shaking and code splitting. Styling was implemented using Tailwind CSS, a utility-first CSS framework that enabled rapid UI development with consistent design patterns through a predefined color palette and spacing scale. The dashboard was deployed on Firebase Hosting, which provides global content delivery network distribution across edge locations and automatic SSL certificate management for secure HTTPS connections.

Figure 19. React frontend architecture showing the component hierarchy, context providers, and data flow patterns.

The Home view serves as the landing page of the ForeWatt dashboard, presenting users with a clean interface for selecting between the consumption and price forecasting models. Two prominently displayed cards provide model selection functionality, each featuring an icon representing the model type, a description of the forecasting capability, and the current best performance metric displayed in a highlighted section. The consumption model card displays the Mean Absolute Error of 892 MWh, while the price model card shows the corresponding MAE of 78.5 TL/MWh. Upon selection, users are automatically navigated to the RealTime view with the selected model pre-configured, streamlining the workflow for users who wish to immediately view current forecasts. The view was designed with responsive layouts that adapt to screen sizes ranging from mobile devices to large desktop monitors, with the two model cards transitioning from a stacked vertical arrangement on mobile to a side-by-side horizontal layout on larger screens.

The RealTime view constitutes the primary interface for monitoring current forecasts, displaying a 24-hour horizon that combines 12 hours of historical actual data from EPIAS with 12 hours of model predictions. The view opens with an AI-generated insight card at the top, providing a natural language summary of current market conditions and notable forecast patterns generated through integration with the Gemini language model. Four key performance indicator cards display summary statistics including average actual value, average forecast value, peak actual with timestamp, and peak forecast with timestamp, enabling rapid assessment of demand levels and timing of peak events. The main interactive chart renders actual data as solid lines and forecast data as dashed lines, with a pivot marker clearly indicating the transition point between historical actuals and predictions. Confidence interval bands were rendered using stacked area series with semi-transparent fills and dashed border lines at the upper and lower bounds, which can be toggled on or off through a control switch. A tabular data section below the chart presents the same information in a sortable table format with columns for timestamp, data type, value, and confidence interval bounds, with options to copy data to clipboard or export as CSV format.

Figure 20. Real-time consumption forecast view showing the 24-hour horizon with actual data, predictions, and confidence intervals.

The Historical view enables exploration of past data with customizable date range selection, supporting both preset ranges and custom date inputs. Preset range options include Latest 3 Days, Latest 7 Days, Latest 15 Days, Latest 1 Month, Latest 3 Months, Latest 6 Months, and Latest 1 Year, with the default selection set to 7 days for optimal initial loading performance. Custom range selection was implemented through datetime-local input fields that allow users to specify precise start and end timestamps down to the minute level. The main visualization displays actual EPIAS data as solid lines and model forecasts as dashed lines, with toggle controls enabling users to show or hide either series independently for focused analysis. Summary statistics cards display key metrics including minimum, maximum, mean, standard deviation, and count for the selected period. Two supplementary charts provide additional analytical perspectives: an hourly patterns bar chart showing average values grouped by hour of day to reveal diurnal consumption cycles, and a value distribution histogram showing the frequency distribution of values with percentile markers at P25, P50, and P75 positions.

Figure 21. Historical analysis view displaying consumption data over a selected time period with actual versus forecast comparison.

The Anomaly view presents statistical anomaly detection results for monitoring unusual patterns in electricity consumption and price data. The detection algorithm employs a configurable sigma threshold approach where data points exceeding the mean by more than the specified number of standard deviations are flagged as anomalies. Users can adjust the sensitivity through a dropdown selector offering 1.5x (high sensitivity), 2x (normal), 2.5x (low sensitivity), and 3x (very low) threshold options, enabling adaptation to different operational contexts and tolerance for false positives. Summary cards display the anomaly rate as a percentage, total anomaly count, maximum deviation observed in sigma units, and total data points analyzed. The main visualization renders a time series line chart with the actual data and horizontal dashed lines indicating the upper and lower bounds, while detected anomalies are highlighted as prominent scatter points with red coloring and shadow effects for visibility. A sidebar panel lists the top anomalies ranked by deviation magnitude, showing the timestamp, value, and sigma multiple for each detected event to facilitate investigation of specific incidents.

Figure 22. Anomaly detection view displaying statistical anomalies in consumption data with configurable sensitivity threshold.

The Compare view facilitates period-over-period analysis through overlaid time series visualizations that enable identification of trends and pattern changes between comparable time periods. Three comparison presets were implemented: Day-over-Day comparing today versus yesterday, Week-over-Week comparing the current week against the previous week, and Month-over-Month comparing the current month against the prior month. Metric cards display comparative statistics including mean difference, peak difference, and volatility measurements for both periods, with percentage change indicators highlighting increases or decreases. The main overlay chart renders both time series on aligned axes, with the current period shown as a solid line with area fill and the comparison period as a dashed line without fill for visual distinction. A secondary day-type comparison chart compares weekday versus weekend consumption profiles using hourly average data, displaying the percentage difference between weekday and weekend means to quantify the day-type effect on demand patterns. A statistics summary table provides side-by-side comparison of minimum, maximum, and standard deviation values for both periods with calculated percentage differences.

Figure 23. Period comparison view showing week-over-week analysis with overlay charts and comparative statistics.

The Alerts view provides a comprehensive management interface for configuring threshold-based alerts on forecast values, enabling proactive notification when predicted or actual values exceed operational limits. The configuration tab displays alert rules as cards, each containing the alert title, description, severity level (critical, warning, or info), enable/disable toggle, and editable threshold value with associated unit and comparison direction. Four default alert configurations were provided: Price Spike Alert triggering when PTF price exceeds 2200 TL/MWh, Negative/Low Price Alert triggering below 100 TL/MWh, Demand Surge Alert triggering above 48000 MWh consumption, and Low Consumption Alert triggering below 25000 MWh. Threshold values can be edited inline by clicking on the value and entering a new number, with save and cancel controls appearing for confirmation. The severity indicator is displayed as a colored badge with appropriate styling for critical (red), warning (amber), and info (blue) levels. Access to the Alerts view was restricted to administrator users through role-based access control implemented in the AuthContext provider.

Figure 24. Alert configuration view showing threshold settings for price and consumption alerts with severity levels.

The Alerts History tab provides a chronological log of triggered alerts, enabling operators to review past events and track system performance over time. Each alert entry displays the alert title, detailed message describing the trigger condition, severity badge, alert type classification, and timestamp in localized format. Unread alerts are visually distinguished with a subtle background highlight and an animated pulse indicator, ensuring new events are immediately noticeable. Action buttons allow users to mark individual alerts as read or delete them from the history log. A refresh button triggers re-evaluation of current data against configured thresholds to identify any new alerts that should be generated. The data status indicator shows the timestamp of the most recent data against which alerts were checked, providing transparency about the currency of alert evaluations. When no alerts have been triggered, a success state is displayed with a checkmark icon and reassuring message indicating all systems are operating within normal parameters.

Figure 25. Alert history view displaying a log of triggered alerts with status tracking and management controls.

An AI-powered chatbot was integrated into the dashboard using the Google Gemini 2.0 Flash model through the @google/genai SDK, providing natural language interaction capabilities for data exploration and insight generation. The chatbot was configured with a domain-specific system prompt that establishes context about the ForeWatt platform, EPIAS data characteristics, typical electricity consumption patterns in Turkey, and the structure of available forecast data. When the chat session is initiated through the header button, current dashboard context including the active model type, latest actual values, forecast predictions, and summary statistics is automatically injected into the conversation to enable contextual responses that reference the specific data currently being viewed. The chatbot interface supports markdown rendering with syntax-highlighted code blocks through the react-markdown library with remark-gfm plugin for GitHub Flavored Markdown support, enabling formatted responses with tables, lists, and code examples. The chat panel can be expanded to full-screen mode for extended conversations, and a fallback mock response system was implemented to provide basic functionality when the Gemini API key is not configured in the environment.

Figure 26. Real-time price forecast view with the AI chatbot panel open, demonstrating contextual conversation capabilities.

The user interface was designed with a glassmorphism aesthetic featuring semi-transparent panels with backdrop blur effects, providing a modern and professional appearance that differentiates ForeWatt from conventional data dashboards. A theme toggle was implemented to support both dark and light modes, with the dark mode set as the default for reduced eye strain during extended monitoring sessions in control room environments. Bilingual support was provided through a LanguageContext provider that manages translations between English and Turkish, enabling seamless language switching without page reload through React context-based state management. The interface was designed to be fully responsive, adapting layouts for desktop, tablet, and mobile screen sizes through Tailwind CSS responsive utility classes with breakpoints at 640px, 768px, 1024px, and 1280px.

Data visualization was implemented using ECharts, a powerful charting library that provides interactive features including zooming, panning, data point tooltips, and chart export capabilities through its built-in toolbox. The RealTimeChart component renders actual data as solid lines and forecast data as dashed lines, with a pivot marker indicating the transition point between historical actuals and predictions at the T-2h boundary accounting for EPIAS data publication delay. Confidence interval bands were rendered using stacked area series with semi-transparent fills and dashed border lines at the upper and lower confidence bounds. The charts automatically adapt to theme changes through a MutationObserver that monitors the document's dark mode class and reinitializes charts with appropriate color schemes when the theme toggles. State management was handled through React hooks with Context providers for global state including authentication status through AuthContext, language preferences through LanguageContext, and real-time data through prop drilling from the App component. API data fetching was implemented with automatic refresh intervals of 60 seconds to keep the real-time view updated with the latest forecast data from the backend API.

# 3. Conclusions

This technical report has presented ForeWatt, a production-ready electricity market forecasting platform designed specifically for the Turkish electricity market. The platform represents a comprehensive solution that integrates advanced machine learning techniques with robust data engineering practices to deliver accurate and reliable forecasts for both electricity consumption and day-ahead prices. Through systematic experimentation and rigorous evaluation, the developed system has demonstrated significant improvements over baseline forecasting methods, establishing its viability for real-world deployment in energy market applications.

The consumption forecasting model achieved a symmetric Mean Absolute Percentage Error of 1.95 percent on the test dataset, representing a 2.8-fold improvement over the seasonal naive baseline. This level of accuracy was attained through careful feature engineering that incorporated temporal patterns, meteorological variables, and calendar effects specific to the Turkish electricity demand profile. The price forecasting model achieved an sMAPE of 11.71 percent, demonstrating a 1.8-fold improvement over the corresponding baseline. Given the inherently volatile and complex nature of electricity price dynamics, this performance represents a substantial achievement in capturing the fundamental drivers of price formation in the Turkish market.

The evaluation of deep learning architectures, including Temporal Fusion Transformers, N-HITS, and PatchTST, revealed important insights regarding the applicability of these methods to medium-scale energy forecasting problems. Despite their demonstrated success in large-scale forecasting competitions, these architectures exhibited significant overfitting tendencies when applied to the available historical data spanning approximately two years. The gradient boosting ensemble approach, specifically the CHybrid V14 architecture combining CatBoost and LightGBM with context-aware error correction, proved to be the most effective methodology for this application domain.

Several technical contributions have been made through the development of this platform. The medallion data architecture, organized into bronze, silver, and gold layers, provides a robust and maintainable framework for data ingestion, cleaning, and feature engineering. This architecture ensures data quality and traceability while facilitating the integration of multiple heterogeneous data sources including EPIAS market data, meteorological observations, and macroeconomic indicators. The feature engineering pipeline combines domain-specific knowledge of electricity markets with automated feature generation techniques, resulting in comprehensive feature sets that capture both physical and economic drivers of consumption and price dynamics.

The production deployment on Google Cloud Run, coupled with the React-based dashboard and Firestore integration, demonstrates the practical applicability of the developed system. The hourly forecast updates, real-time visualization capabilities, and AI-powered chatbot interface provide stakeholders with actionable insights for market participation decisions. The anomaly detection module based on Isolation Forest algorithms further enhances the platform's utility by identifying unusual patterns that may require additional attention from market analysts.

Several directions for future work have been identified through this research. The expansion of the platform to cover additional electricity markets, particularly those in neighboring countries with interconnected grid infrastructure, represents a natural extension of the current capabilities. The integration of real-time news sentiment analysis could enhance price forecasting accuracy during periods of market stress or significant policy announcements. The implementation of automated model retraining pipelines would ensure that forecast accuracy is maintained as market dynamics evolve over time. Furthermore, the development of probabilistic forecasting capabilities through conformal prediction methods would provide decision-makers with calibrated uncertainty estimates essential for risk management applications. Enhanced anomaly detection incorporating temporal dependencies and market regime identification represents another promising avenue for improving the platform's analytical capabilities.

# References

[1] Lim, B., Arık, S. Ö., Loeff, N., & Pfister, T. (2021). Temporal Fusion Transformers for interpretable multi-horizon time series forecasting. International Journal of Forecasting, 37(4), 1748-1764.

[2] Challu, C., Olivares, K. G., Oreshkin, B. N., Ramirez, F. G., Canseco, M. M., & Dubrawski, A. (2023). N-HiTS: Neural hierarchical interpolation for time series forecasting. Proceedings of the AAAI Conference on Artificial Intelligence, 37(6), 6989-6997.

[3] Nie, Y., Nguyen, N. H., Sinthong, P., & Kalagnanam, J. (2023). A time series is worth 64 words: Long-term forecasting with transformers. International Conference on Learning Representations (ICLR).

[4] Vaswani, A., Shazeer, N., Parmar, N., Uszkoreit, J., Jones, L., Gomez, A. N., Kaiser, Ł., & Polosukhin, I. (2017). Attention is all you need. Advances in Neural Information Processing Systems, 30.

[5] Prokhorenkova, L., Gusev, G., Vorobev, A., Dorogush, A. V., & Gulin, A. (2018). CatBoost: Unbiased boosting with categorical features. Advances in Neural Information Processing Systems, 31.

[6] Ke, G., Meng, Q., Finley, T., Wang, T., Chen, W., Ma, W., Ye, Q., & Liu, T. Y. (2017). LightGBM: A highly efficient gradient boosting decision tree. Advances in Neural Information Processing Systems, 30.

[7] Chen, T., & Guestrin, C. (2016). XGBoost: A scalable tree boosting system. Proceedings of the 22nd ACM SIGKDD International Conference on Knowledge Discovery and Data Mining, 785-794.

[8] Weron, R. (2014). Electricity price forecasting: A review of the state-of-the-art with a look into the future. International Journal of Forecasting, 30(4), 1030-1081.

[9] Hong, T., & Fan, S. (2016). Probabilistic electric load forecasting: A tutorial review. International Journal of Forecasting, 32(3), 914-938.

[10] Lago, J., Marcjasz, G., De Schutter, B., & Weron, R. (2021). Forecasting day-ahead electricity prices: A review of state-of-the-art algorithms, best practices and an open-access benchmark. Applied Energy, 293, 116983.

[11] Ziel, F., & Weron, R. (2018). Day-ahead electricity price forecasting with high-dimensional structures: Univariate vs. multivariate modeling frameworks. Energy Economics, 70, 396-420.

[12] Nowotarski, J., & Weron, R. (2018). Recent advances in electricity price forecasting: A review of probabilistic forecasting. Renewable and Sustainable Energy Reviews, 81, 1548-1568.

[13] Liu, F. T., Ting, K. M., & Zhou, Z. H. (2008). Isolation forest. Proceedings of the 2008 Eighth IEEE International Conference on Data Mining, 413-422.

[14] EPIAS. (2024). EPIAS Transparency Platform documentation. Energy Exchange Istanbul. https://seffaflik.epias.com.tr/

[15] Kucukali, S., & Baris, K. (2010). Turkey's short-term gross annual electricity demand forecast by fuzzy logic approach. Energy Policy, 38(5), 2438-2445.

[16] Open-Meteo. (2024). Open-Meteo Weather API documentation. https://open-meteo.com/

[17] Central Bank of the Republic of Turkey. (2024). Electronic Data Delivery System (EVDS). https://evds2.tcmb.gov.tr/

[18] Ramírez, S., & FastAPI Contributors. (2024). FastAPI: Modern, fast web framework for building APIs with Python. https://fastapi.tiangolo.com/

[19] React Contributors. (2024). React: A JavaScript library for building user interfaces. https://react.dev/

[20] Google Cloud. (2024). Cloud Run documentation. https://cloud.google.com/run/docs

[21] Firebase. (2024). Firebase documentation. https://firebase.google.com/docs

[22] Pedregosa, F., Varoquaux, G., Gramfort, A., Michel, V., Thirion, B., Grisel, O., ... & Duchesnay, É. (2011). Scikit-learn: Machine learning in Python. Journal of Machine Learning Research, 12, 2825-2830.

[23] McKinney, W. (2010). Data structures for statistical computing in Python. Proceedings of the 9th Python in Science Conference, 56-61.

[24] Vovk, V., Gammerman, A., & Shafer, G. (2005). Algorithmic learning in a random world. Springer Science & Business Media.

[25] Romano, Y., Patterson, E., & Candès, E. J. (2019). Conformalized quantile regression. Advances in Neural Information Processing Systems, 32.

[26] Korkut, R. (2024). eptr2: Python client for EPIAS Transparency Platform. https://github.com/Rogerspy/eptr2

# Appendix

A. Feature Specifications

Complete feature lists for both consumption and price forecasting models are maintained in the project repository. The consumption model utilizes 23 features organized into four categories: temporal lag features capturing historical consumption patterns at 24-hour, 48-hour, and 168-hour intervals; rolling statistical features including mean and standard deviation computed over 24-hour windows; meteorological features derived from weather observations across 12 Turkish cities; and calendar features encoding hour-of-day, day-of-week, month, weekend indicators, and Turkish public holidays. The price model employs 33 features encompassing price lags and rolling statistics, market fundamental indicators such as thermal gap and renewable saturation, system operation metrics including load factor and reserve margin ratios, and specialized features for solar generation patterns and price volatility dynamics.

B. Hyperparameter Configurations

The CatBoost models were trained with the following key hyperparameters: learning rate of 0.05, maximum depth of 8, L2 regularization coefficient of 3.0, and 2000 iterations with early stopping patience of 100 rounds. The LightGBM component of the price ensemble utilized a learning rate of 0.03, maximum depth of 10, number of leaves set to 64, and feature fraction of 0.8 for regularization. Both models employed symmetric quantile loss functions for training the prediction interval bounds. The Isolation Forest anomaly detection model was configured with 200 estimators, contamination parameter of 0.05, and bootstrap sampling enabled. All hyperparameters were determined through grid search with 5-fold time series cross-validation on the training dataset.

C. API Endpoint Reference

The ForeWatt API exposes several endpoints for forecast retrieval and system monitoring. The health check endpoint (GET /health) returns the current system status and version information. The forecast trigger endpoint (POST /forecast) initiates the forecast pipeline and is invoked hourly by Google Cloud Scheduler. The latest forecast endpoint (GET /forecast/latest) returns the most recent 24-hour forecasts for both consumption and price models. Separate endpoints are available for retrieving price forecasts (GET /forecast/price) and consumption forecasts (GET /forecast/consumption) individually. Real-time data streaming is supported through the Firestore-backed endpoints (GET /api/realtime/{model}), while historical data access is provided via Parquet-based endpoints (GET /api/historical/{model}). Anomaly detection results are accessible through dedicated endpoints (GET /api/anomaly/{model}) that return identified anomalies with associated confidence scores and feature contributions.

All API responses follow a standardized JSON schema with fields for timestamp, model version, forecast values, and optional confidence intervals. Authentication is handled through API keys for production deployments, while the development environment supports unauthenticated access for testing purposes. Rate limiting is implemented at 1000 requests per hour per client to ensure fair resource allocation among users.